### Calculation For Design of Column [column_id]

Data

- f’c = [f_c] Ksi

- fy = [f_y] Ksi

- Pu = [Pu] kips (From Table )

- Ø = [phi]

- K = [k_factor]

For Percentage of Steel:

Assuming,  ρ = [ρ]% or 0.01

Calculating Gross Area (Ag):

Pu = Øk [0.85 f’c (Ag – As) + As fy]

[Pu] = ([phi]) ([k_factor]) [0.85 ([f_c]) (Ag – [ρ]xAg ) + [ρ]x Ag ([f_y])]

Ag = [Ag_calculated] in²

Calculating Dimensions of Column:

Assuming a column width L1 = [one_side] in.

L2 = Ag / L1

L2 = [Ag_calculated] / [one_side]

L2 = [second_side_calculated]

L2 ≈ [second_side] in

Hence,

Ag (provided) = [one_side] x [second_side]

Ag (provided) = [Ag_provided] in²

Calculating Area of Steel (As):

As = 0.01 x Ag (provided)

As = 0.01 x [Ag_provided]

As = [As_calculated] in²

Calculating Main Reinforcement:

Assuming #[bar_no] bar as a main longitudinal bar

No. of Bars = As (provided) / Ab

No. of Bars ≈ [no_of_bars] bars

Calculating Area of Steel (As):

As (provided) = No. of Bars x Area of bars

As (provided) = [no_of_bars] x (3.142/4 x ([bar_no]/8))²

As (provided) = [As_provided] in²

Calculating Spacing b/w Ties:

- 48 (Dia of tie bar) = 48 (3/8) = [tie_spacing_1]”

- 16 (Dia of longitudinal bar) = 16 ([bar_no]/8) = [tie_spacing_2]”

- Least dimension of Column = [tie_spacing_3]”

Therefore, ties are to given at #3@[final_tie_spacing]”c/c

Calculating Nominal Capacity:

Ø Pn = Øk [0.85 f’c (Ag (provided) – As (provided)) + As (provided) x fy]

Ø Pn = ([phi]) ([k_factor]) [0.85 ([f_c]) ([Ag_provided] – [As_provided]) + [As_provided] x ([f_y])]

Ø Pn = [nominal_capacity] Kips

Hence, the designed column has nominal capacity of [nominal_capacity] kips

Reinforcement Details:

    - Column Dimensions: [one_side]” × [second_side]”

    - Number of Bars: [no_of_bars]#[bar_no] bars

    - Tie Spacing: #3@8” c/c

    - Nominal Capacity: [nominal_capacity] Kip